| VESSEL: |  |
| --- | --- |
| DATE: |  |
| PLACE: |  |

PSC INSPECTION PREPARATION CHECKLIST

| Distribution List By Rank | Α. Master (SSO) | Β. 3rd Officer | C. 2nd Officer | D. Ch. Officer | E. Chief Engineer | F. Electrician |
| --- | --- | --- | --- | --- | --- | --- |
| Pages  | 1, 2, 3 | 4, 5, 6 | 7, 8, 9 | 10, 11, 12, 13 | 14, 15, 16, 17 | 18 |

| Rank | Name & Signature | Rank | Name & Signature |
| --- | --- | --- | --- |
| Master: |  | Ch. Engineer: |  |
| Ch. Mate: |  | 2nd Engineer: |  |
| 2nd Officer: |  | 3rd Engineer: |  |
| 3rd Officer: |  | Electrician: |  |

| A. Master (SSO) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Item |  | Check | X= OK, N= Not Checked, N/A= Not applicable, R = REMARK – explain |  |
| 1. Documentation |  |  |  |  |
| 1. | Class and Flag Certificates | - Printed Certificates available with valid endorsements (Class) - List with due dates updated in the folder. |  |  |
| 2. | Enhanced Survey Program (ESP) file | a. Check Executive Hull Summary report. Issued every 5 years for vessel until 10 years old and thereafter, every 2,5 years. |  |  |
| 3. | Miscellaneous Certificates (Original to be available) | - Certificates from last annual and 5-yearly inspection of Lifeboats and Davits. |  |  |
|  |  | - EPIRB Test Certificate. |  |  |
|  |  | - VDR Service Certificate. |  |  |
|  |  | - LRIT Test records. |  |  |
|  |  | - Certificates for Ropes. |  |  |
|  |  | f. GMDSS checklist |  |  |
|  |  | g. Certificates for Safety Equipment Service [Fire extinguishers (CO2 Gas Cylinder, Air foam, etc.), EEBDs, Life Rafts]. |  |  |
|  |  | h. Calibration Certificate for Multi Gas meters. |  |  |
|  |  | i. Type approval certificate for incinerator. |  |  |
|  |  | j. Type approval certificate for 15ppm alarm. |  |  |
|  |  | k. Copy of P&I club coverage regarding MLC 2006, 2014 amendments to be posted in Mess rooms (applicable since 18/1/2017) |  |  |
| 4. | Officers and Crew Certificates | - Medical Certificates |  |  |
|  |  | - Flag Endorsements |  |  |
|  |  | - CoC |  |  |
|  |  | - STCW Endorsement |  |  |
|  |  | - ECDIS Generic & Specific Training Cert (Bridge officers) |  |  |
|  |  | - Employment Contract valid & signed by crew |  |  |
| 5. | Continuous Synopsis Records (CSR) | CSR documents signed by Master and CSR amendment forms exist onboard. |  |  |
| 6. | Cargo Gear Book | Properly endorsed by class. |  |  |
| 7. | Record of Drills | Drills/ Training carried out and logged as per Company’s Drill’s Matrix. |  |  |
| 8. | Abandon Drills | - Abandon ship drill every month. - Lowering and manoeuvring to water of Lifeboat and Rescue Boat every 3 months. - Free-fall lifeboat only: At intervals of not than 6 months, a simulated launching to be performed in line with manufacturer’s instructions. |  |  |
|  |  | d. Crew able to perform abandon drill successfully. As per PSC procedures, on cargo ships, it is required that lifeboats and davit-launched life rafts are capable of being launched within a period of 10 minutes. |  |  |
| 9. | Fire Drills | a. Abandon ship drill every month. |  |  |
|  |  | b. Crew able to perform a fire drill successfully. Fire squad properly dressed, capable of handling VHFs, each person knows their duties. |  |  |
| 2. ISPS |  |  |  |  |
| 1. | CSO contact details posted | Updated company’s relevant poster to exist onboard. |  |  |
| 2. | SSO is appointed | Crew knows that SSO is the Master and Deputy SSO the Ch. Mate. |  |  |
| 3. | Access to the ship | Gangway watchman is familiar with gangway duties/ MARSEC levels/ etc. |  |  |
| 4. | Visitor’s Book - cards | Visitor log properly filled, and all crew have their individual ISPS cards. |  |  |
| 5. | Restricted Areas Secured | All external doors of accommodation/ F’cle/ Mast Houses secured. |  |  |
|  |  | No padlocks used from door’s internal side on any door onboard. |  |  |

| B. Third (3rd) Officer |  |  |  |
| --- | --- | --- | --- |
| Item |  | Check | RATE: X= OK, N= Not Checked, N/A= Not applicable, R = REMARK – explain |
| 1. Accommodation/ Safety |  |  |  |
| 1. | Muster List | Updated list of ship Muster station(s), including deputies for each position. |  |
| 2. | Safety Cards | Safety card /emergency duties posted in each cabin. |  |
| 3. | Fire Doors | Self-closing fire doors operational. No hold-back ropes/hooks fitted. Usually placed on the galley doors. |  |
| 4. | Fire Control Plan | a. Properly posted in alleyways. |  |
|  |  | b. Permanently stored in watertight cases outside the accommodation main entrances. (Port & Starboard). |  |
| 5. | Training Manual | Provided in each crew mess/recreation room. Instructions for life-rafts correspond to actual maker of life-rafts onboard. |  |
| 6. | Means of Escape | Marking (IMO symbol) of escape routes and unobstructed (from stores). |  |
| 7. | Distress Flares | At least 12 valid parachute rocket signals available. |  |
| 8. | Lifeboats | a. Good condition of hull, call sign/name visible, attachments, and reflective tapes. |  |
|  |  | b. Good condition of propeller, rudder, stock, and stern frame |  |
|  |  | c. Engine in good condition tested/recorded. |  |
|  |  | d. Instructions of starting / stopping the engine available. |  |
|  |  | e. Grab lines / painters in good condition. |  |
|  |  | f. Magnetic compass free of air bubble / No excessive error. |  |
|  |  | g. Embarkation ladder / brackets / shackles in good condition |  |
|  |  | h. Instructions of on load release system operation available. |  |
|  |  | i. Lock/unlock position clearly marked on the on-load release indicator. |  |
|  |  | j. Inventory to exist. No expired equipment/stores |  |
| 9. | Immersion suits | a. Distributed in proper locations, as per LSA Plan |  |
|  |  | b. Glue on zips intact, zips & closures in good condition. |  |
| 10. | Line-throwing appliances | a. 4 rockets capable of carrying line at least 230m. |  |
|  |  | b. Proper instruction available. |  |
| 11. | Lifejackets | a. Good condition, correct number and marking. |  |
|  |  | b. Lights, reflective tapes, batteries and whistles in good condition. |  |
| 12. | Life buoys | a. Good condition, visible number and marking. |  |
|  |  | b. Lights, quick release and lifelines in good condition. |  |
| 13. | Portable foam applicator unit | a. Checking the air-foam nozzle, portable tank of foam making liquid, and one spare tank. Inspection records. |  |
|  |  | b. Validity of foam liquid analysis is for New: 3 years and thereafter: 1 year |  |
|  |  | c. Condition of stowage container in good order. |  |
| 14. | Fire extinguishers (CO2, foam, dry powder, etc.) | a. Visually in good condition, periodical inspection records last service date stamp available. |  |
|  |  | b. Correct quantity, placed, readily available as per fire plan. |  |

| C. Second (2nd) Officer |  |  |  |  |
| --- | --- | --- | --- | --- |
| Item |  | Check |  | RATE: X= OK, N= Not Checked, N/A= Not applicable, R = REMARK – explain |
| 1. Bridge |  |  |  |  |
| 1. | Radar | a. In good operating condition/ Screen without “lines”. |  |  |
|  |  | b. Enter Radar performance into the logbook every watch. |  |  |
|  |  | c. Use speed from speed log for ARPA. |  |  |
| 2. | Magnetic Compass | a. Easily Readable / No air bubble. |  |  |
|  |  | b. Emergency lighting operational. |  |  |
|  |  | c. Last calibration (updated deviation chart). |  |  |
| 6. | Compass error logbook | Compass errors to be calculated once per watch, weather permitting. Check deviation with the compass calibration chart. Difference should not be greater than 5 deg. |  |  |
| 7. | Lights, shapes, sound signals | In good condition. |  |  |
| 8. | Echo sounder | Functioning correctly. If memory is available, keep the approaches to last port. |  |  |
| 9. | INMARSAT–C/EGC | In good operating condition. Nav-area, ECG Warnings. Special attention to Australian warnings. |  |  |
| 10. | Radio / GMDSS | a. Installations (MF, MF/HF) in operating condition. |  |  |
|  |  | b. GMDSS batteries charged. |  |  |
|  |  | c. DSC distress and cancellation of distress call to be demonstrated by bridge officers. |  |  |
| 11. | GMDSS logbook | a. Name of the GO on the relevant page (To be the same as in Muster list). |  |  |
|  |  | b. Equipment test records such as daily/ weekly/ Monthly including batteries, EPIRB test. |  |  |
|  |  | c. Daily noon entry for ship’s position/ Summary of DSC calls. |  |  |
| 12. | VHF | Good operating condition / spare batteries / validity. |  |  |
| 13. | MF/ HF | In good operating condition including DSC performance. |  |  |
| 14. | EPIRB | Validity of battery and hydrostatic release, updated maintenance records. |  |  |
| 15. | Ship’s Radar transponder (SART) | Correctly located, battery not expired. |  |  |
| 16. | BNWAS | 1st Stage Alarm operational (and then 2nd stage etc.)- “ON” mode (not “AUTO”) always activated at sea. |  |  |
| 17. | NAVTEX | NAVTEX warnings should be monitored by the officer on watch at the time of receipt. Ensure the warnings received they have been checked starting with those are relevant with current voyage. |  |  |
|  |  | Correctly programmed to receive from appropriate stations. Visible printouts (if Printer exists). |  |  |
| 18. | AIS | Correctly programmed to transmit the vessel’s activity. |  |  |
| 19. | Engine Control Printer (if fitted) | Where applicable: Evidence that Astern Movement was carried out before arrival. |  |  |
| 20. | Air horn | In good condition. |  |  |
| 21. | Navigation lights | Ensure with electrician (para. F of this checklist) they are in good condition/alarms in order. |  |  |
| 22. | Shore based maintenance agreement | Properly posted in Radio Room for GMDSS and EPIRB. |  |  |
| 23. | Bridge Logbook | English Language in the logbook. |  |  |
|  |  | Include Navigation Activities from pilot to berth. |  |  |
|  |  | Hardcopy logbook: Use of “blanko” (correction fluid) is not permitted for correction in official logbooks. Correction should be done by means of a crossing line so that the original entry can be read. Then you write the correction and your initials. |  |  |
| 24. | Navigation Procedures / UKC | a. “Pre-Arrival”/ “Pre-Departure”/ “Change over watch” checklists posted at bridge, reviewed each time and logbook entry is made. |  |  |
|  |  | b. Officers to be familiar with Under Keel Clearance policy in Open Sea, narrow waters, at Port limits (Refer to Passage Plan SMS form). |  |  |
|  |  | c. Master’s Standing Orders (signed by Master and Navigation Officers). |  |  |
|  |  | d. Master’s Night Order Book updated daily. |  |  |
| 25. | Charts/ ECDIS / Nautical Publications | 1. Either hardcopy or electronical charts/publica tions. | a. Evidence that position fixing frequency is as required by the passage plan. |  |
|  |  |  | b. From pilot station (with pilot onboard) to berth enter way points, courses, and distances. |  |
|  |  |  | c. Visual and radar position fixing, and monitoring techniques should be used whenever possible. |  |
|  |  |  | d. Passage Plan from Berth to Berth. |  |
|  |  |  | e. All ITU publications up-to-date |  |
|  |  | 2. ECDIS | - All electronic charts (ENCs) for the intended voyage to be available and corrected to the latest version. - ADP and e-NP to be updated to the latest week. |  |
|  |  |  | c. All navigation officers to be familiar with Safety contour and other safety settings such as off track (XTE), look ahead, safety depth etc. |  |
|  |  |  | d. AIO symbols. Know to activate/de-activate on ECDIS- explain symbols. |  |
|  |  | 3.Paper Charts (If applicable) | a. Cumulative list of Notices to Mariners available. |  |
|  |  |  | b. Applicable Temporary and Preliminary corrections have been done. |  |
|  |  |  | c. Navigation Warnings and NAVTEX warnings are marked on the Charts. (Paper charts). |  |
|  |  |  | d. Parallel Indexing to be used when possible. |  |
|  |  |  | e. T&P NTM available and updated. |  |
| 26. | Bridge Wings | - Gyro Repeaters indication is correct. |  |  |
|  |  | - RPM/ Anemometer/ Clinometer repeaters in good working order. |  |  |
|  |  | - Man-Overboard Buoys of correct weight. |  |  |
|  |  | - MOB ropes tidy. |  |  |
| 27. | Hospital | - No expired Medicines. |  |  |
|  |  | - Oxygen Resuscitators in place. |  |  |
|  |  | - Medical log available in hospital. |  |  |
| 28. | Bridge visibility | Bridge windows in good condition/ window wipers and water sprinkler functional (not blocked). |  |  |

| D. Chief Officer |  |  |  |
| --- | --- | --- | --- |
| Item |  | Check | RATE: X= OK, N= Not Checked, N/A= Not applicable, R = REMARK – explain |
| 1. Documents |  |  |  |
| 1. | Garbage Record Book/ Plan | Properly recorded and updated. Check entries for cargo residues or cargo wash-water in Special Areas. |  |
| 2. | SOPEP | - Updated “Contact points” and “Port Contacts”. - Updated list of pollution prevention equipment to be included in SOPEP room and SOPEP manual. |  |
| 3. | Rest Hours (DECK Department) | a. Rest hour forms (Deck Dpt) printed for the last full month and completed in the computer until the latest day of current month. : Records of Rest Hours. Please cross check with logbooks so that there are no discrepancies (e.g., Master on the bridge during approaches, Chief Officer forward during anchoring, Crew members appear on “rest” during drill time). |  |
| 4. | Work Permit System (WPS) | Entry into enclosed spaces checklist issued for any enclosed space, tank inspection. Working Overside/ aloft. The respective Risk Assessment reviewed |  |
| 5. | Loading / stability manual | Approved and with intact stability information |  |
| 2. Deck walk |  |  |  |
| 6. | Main + Superstructure decks | No corrosion / buckling or other damage. |  |
| 7. | Lifeboat Davits | a. Good condition of structure, blocks, falls, pad-eyes, links, fastening and all other outfitting. Well maintained. |  |
|  |  | b. Launching instructions underneath emergency light. |  |
|  |  | c. Good Condition of boat lights by emergency power. |  |
|  |  | d. Operation of stowage limit switch or air cut-off valves. |  |
|  |  | e. Good Condition of brake (Winch). |  |
| 8. | Rescue boat | a. Good condition of hull, attachments |  |
|  |  | b. Good condition of engine, tested/recorded. |  |
|  |  | c. Inventory as required. |  |
|  |  | d. Good condition of hooks & attachments |  |
|  |  | e. Control lever operable (for 6-person R/B), greased and not frozen. |  |
| 9. | Life-rafts | a. Launching instructions underneath emergency light. |  |
|  |  | b. Hydrostatic release Unit not expired / Weak link correctly fitted. |  |
|  |  | c. Updated vessel’s details/crew list. |  |
|  |  | d. Proper stowage. |  |
|  |  | e. Good condition of embarkation ladder. |  |
| 10. | Means of escape | IMO symbol in place as per LSA plan. Not obstructed with stores/items. |  |
| 11. | Railings/ Bulwarks | Not bented/ corroded. |  |
| 12. | Lifelines, gangways, passages | No corrosion / wastage corrosion, missing parts, or holes. |  |
| 13. | Pilot ladders / accommodation ladders | a. Good operational condition. Safe access. Pilot access point free of wastage. |  |
|  |  | b. Safety net in good condition and correctly fitted. |  |
| 14. | Draft/ Plimsol Marks | Readable, free from rust (port and starboard) |  |
| 15. | Cargo and other hatchways/ access | a. No cracks, buckling on h/coamings, stays etc., covers in good condition |  |
|  |  | b. Hatch cover stoppers always in place when at port |  |
|  |  | c. Non-return drain valves in good condition (not blocked, missing). |  |
|  |  | d. Hatch cover hydraulic jacks without hydraulic oil leaks. |  |
|  |  | e. Hatch accesses to have safety pins available (no bolts!) |  |
| 16. | Fixed fire extinguishing arrangement in E/R (e.g., CO2) | a. Piping in lines in good order, without leakage or no heavy corrosion. |  |
|  |  | b. Test of alarm (if possible). |  |
| 17. | Accommodation external watertight Doors | Doors, gaskets and handles in good condition, Door edges not wasted. “O” (open) & “S” (shut) marked. |  |
| 18. | Crane Mast house external watertight doors |  |  |
| 19. | Tank Manholes on deck | No Corrosion, handles/ no missing bolts/ marking in good condition. |  |
| 20. | Tank Air vents | Good condition, no corrosion, internal ball/disc ok. Clearly marked. |  |
| 21. | Fire Line | a. No leakages/ wastage, hydrant valves in order |  |
|  |  | b. Fire hoses not leaking / nozzles and F-keys (where fitted) not missing from hydrant boxes. |  |
| 22. | De-watering system | In good working condition/ Valves operable/ alarms in order. |  |
| 23. | Remote control of valves (hydraulic) | In good working order. “Open”/ “Closed” indications visible. |  |
| 24. | Mooring arrangements | a. Good condition of mooring ropes, anchoring devices, |  |
|  |  | b. Platforms, gratings in good condition, no wastage. |  |
|  |  | c. Brake Lining with adequate thickness. |  |
|  |  | d. Clutch pins to be fitted to all winches when vessel is moored. |  |
|  |  | e. Winches clearly marked and in good condition. |  |
|  |  | f. No hydraulic oil leakages. |  |
|  |  | g. Hydraulic levers free to move, levers’ stoppers in order. |  |
| 25. | Masts, posts, crane jibs | Good condition, no serious wear/corrosion or damage. |  |
| 26. | Emergency Towing Arrangements | ΕΤ Booklet available at Ship’s office, bridge and forecastle space. |  |
| 27. | International shore connections | In good condition, boxes with gaskets, bolts and spanners in position |  |
| 28. | Electric cable lines/pipes | Good condition, no heavy wastage, corrosion, cracks. |  |
| 29. | Oxygen and Acetylene Room | a. Flash Back arrestors /Back Fire valves in place on both acetylene and oxygen. |  |
|  |  | b. Regulators and gauges in good condition. |  |
|  |  | c. Spare bottles to be secured by U bolts. |  |
|  |  | d. Hoses in good condition and kept disconnected when not in use. |  |
| 30. | Sanitary/ Shower Facilities | a. Toilets bowl/ Urinals clean, unblocked and not damaged. |  |
|  |  | b. Flushing valves in good condition and operable. Adequate water pressure on bridge/ upper decks. |  |
|  |  | c. Clean floor. No broken tiles. No odours. |  |
|  |  | d. Hot and cold water on taps of common lavatories. |  |
| 31. | Ventilation, heating, lighting | In good working condition and clearly marked. Air Ventilators grilles in accommodation free from dust/ muck. |  |
| 32. | Laundry + Drying room | Clean and tidy. Clothes properly placed for drying. No other stores/items to exist in drying room. |  |
| 33. | Galley + Mess rooms (Chief Cook) | a. Clean and tidy. No utensils/ food on the floor. |  |
|  |  | b. Galley (internal) hood & filters clean. |  |
|  |  | c. Galley (external) air vent grating free of grease. |  |
|  |  | d. Fire extinguisher(s) and Fire blankets in position, personnel familiar with their use. |  |
| 34. | Cranes & davits | SWL marked/ Crane hook safety lock in place. |  |
| 35. | Bunker manifold | In good condition. Max working pressure and date of test marked. |  |
| 36. | Fuel Tank air vents | a. Clearly Marked with tank’s ID |  |
|  |  | b. wire mesh (spark arrestors) in place and not corroded |  |
|  |  | c. Save-all boxes Clean. Capacity to be marked. |  |

| D. Chief Officer |  |  |  |
| --- | --- | --- | --- |
| Item |  | Check |  |
|  |  | d. Drain plugs of boxes in position. |  |
| 37. | Paint Store | a. Paint drums securely lashed, stored lower than the sprinkler. |  |
|  |  | b. Ventilation open/ Ventilation is adequate/ ventilation capable of closing |  |
|  |  | c. Fire-fighting system tested in order. |  |
|  |  | d. Materials Safety Data Sheets (MSDS) available for all paints. |  |
|  |  | e. Protection of lights in position. |  |
| 38. | Forecastle space | a. Clean & tidy/ all stores securely lashed. |  |
|  |  | b. No paint drums stored in the forecastle. |  |
|  |  | c. Forecastle doors/ gaskets in good condition. |  |
|  |  | d. LSA not blocked by stores. |  |
|  |  | e. Emergency Towing Booklet available in F’cle space. |  |
|  |  | f. Rope hatch and gaskets in good condition. |  |

| E. Chief Engineer |  |  |  |
| --- | --- | --- | --- |
| Item |  | Check | RATE: X= OK, N= Not Checked, N/A= Not applicable, R = REMARK – explain |
| 1. Documentation |  |  |  |
| 1. | Logbooks/ Forms | a. Use of “blanko” (correction fluid) is not permitted for correction of official logbooks/ oil record books etc. Correction should be done by means of a straight line so that the original entry can be read. Then, you write the correction and your initials. |  |
|  |  | b. Records of Low Sulphur fuel change-over in place. The time required for change over time to be printed from Lloyds “FOBAS calculator” form and kept with LS change over forms/ logbook. If FOBAS calculator is missing, please ask it from HSQE dpt. |  |
|  |  | c. Oil Record Book Properly recorded and updated. |  |
|  |  | d. NOx Technical File + Engine Parameters logbook. Properly recorded and updated. (For vessels built on or after 01/01/2000) |  |
|  |  | e. Rest hour forms (ENGINE dpt) printed for the last full month and completed in the computer until the latest day of current month. Records of Rest Hours. Please cross check with logbooks so that there are no discrepancies (e.g., Chief or Second Engineer in charge of bunkering operations) |  |
| 2. Engine Room |  |  |  |
| 2. | Overall, of Engine Room | a. No oil / water leakages. No oily rags dropped here and there in ER. |  |
|  |  | b. Bilges clean, free from rust/ oily wastes. |  |
|  |  | c. A-60 Insulation on E/R Bulkhead, hot surfaces/pipes (above 220° C) in place. |  |
|  |  | d. All Counters and Gauges visible and in good working order. |  |
|  |  | e. All lights/covers and guards in good condition. |  |
|  |  | f. Self-closing devices for sounding pipes and sight glasses in order. Free to move and close. |  |
|  |  | g. Tanks level glasses auto close valves in “closed” position. |  |
|  |  | h. Spare parts properly secured/lashed. |  |
|  |  | i. Quick closing valves in working condition. No temporary blocks/bolts to force valves being in “open” position. |  |
|  |  | j. Piping without soft patches (corrosion) / doublers / clamps. No cement boxes fitted. |  |
| 3. | Main engine | - Operate satisfactorily. No leakage of fuel/ oil / water. |  |
|  |  | - Safety Alarms function properly. |  |
|  |  | - Jacketed High Press. FO pipes in order. No leaks. |  |
|  |  | - F.O. leak alarm in good order. |  |
|  |  | - Oil mist detector (If fitted) works satisfactorily and able to be tested by simulation. |  |
| 4. | Auto control system (If fitted) | a. Engine telegraph in order. |  |
|  |  | b. Alarm system/extensions/printer in order. |  |
|  |  | c. Console and all indications in order (lights, gauges, etc.). |  |
| 5. | Auxiliary engines | a. Operate satisfactorily. No leakage of fuel/ oil / water. |  |
|  |  | b. Fuel Oil leakage Alarm and other Safety Alarms function properly. |  |
|  |  | c. Remote control functions work properly. |  |
|  |  | d. FO pipes in order. |  |
| 6. | Auxiliary Boiler | - Operate satisfactorily. No leakage of oil / water / steam. |  |
|  |  | - Safety Alarms (low level, low-low, etc.) function properly. |  |
|  |  | - Remote control functions properly. |  |
|  |  | - Pressure gauges in good order, calibrated every year. |  |
|  |  | - Water level gauges in good order. |  |
| 7. | Stern Tube Seals | No leakage of oil / Sea water. |  |
| 8. | Bilge pumping arrangements | Bilge pump and bilge high level alarm in order. |  |
| 9. | Retention of oil on board | Sludge tanks capacity and arrangement / Certificate for delivery to shore facilities available in ORB. |  |
| 10. | Oily water separator | a. Operates satisfactorily. No visible oil in discharged water. No illegal by-pass. |  |
|  |  | b. No heavy corrosion or holes on the outer casing. |  |
|  |  | c. Operation of valves in good order. |  |
|  |  | d. Pressure gauges in order. |  |
|  |  | e. Sampling test for filtered water from test cock. |  |
|  |  | f. Operating instructions posted. |  |
| 11. | 15 ppm arrangements (Oil Content meter) | a. Operating condition and auto-stop in order. Alarm function. |  |
|  |  | b. Ships equipped with MEPC.107 (49) OCM [see IOPP supplement] Oil Content Meters (OCM) must ensure that the Engine Room staff can retrieve the “historical data” upon request by PSC authorities. All engine room alarm history, Oil Record Book (ORB) entries, OCM history and tank soundings must match with respect to dates and tank levels. Any deviation must be investigated, corrected, and if necessary, reported to flag. All OCM seals must be intact and not tampered. |  |
| 12. | Engine Room Workshop | a. Clean/tidy with sufficient lighting, Free entrances, and escapes. |  |
|  |  | b. Protective Plexiglas in place on grinding wheel / lathe machine etc. |  |
|  |  | c. First aid instructions for electric shock in place near the welding machine. |  |
| 13. | Incinerator | a. Operates satisfactorily. |  |
|  |  | b. Operation instruction card placed in order. |  |
|  |  | c. Alarms in order. |  |
| 14. | Steering gear | a. Operates satisfactorily οn “AUTO” and “Emergency” mode. |  |
|  |  | b. No leakage / oil spill, corrosion structure defects. |  |
|  |  | c. Hydraulic oil tank to be filled, low level alarms in order. |  |
|  |  | d. Gyro repeater with correct indication. |  |
|  |  | e. Crew members familiar with emergency steering operation |  |
|  |  | f. Emergency Steering instructions posted. |  |
| 15. | Chemicals | a. Securely lashed, store separate acids for alkalis to the extent possible. |  |
|  |  | b. Goggles / boots / gloves / eye wash exist at chemical store. Do not use chipping goggles (the ones with little holes). |  |
|  |  | c. Materials safety data sheets for all stored chemicals |  |
| 16. | Duct keel (if fitted) | Clean and tidy. |  |
| 17. | Main fire pump(s) | a. Operate satisfactorily with adequate pressure. |  |
|  |  | b. Pressure gauges in order. |  |
|  |  | c. No leaks. |  |
| 18. | Work Permit System | a. Hot Work permit required for work(s) outside the Engine room Workshop. Respective Risk Assessment has been reviewed. |  |
|  |  | b. Enclosed Entry Permit required for Crank case inspection. |  |
| 19. | Emergency generator | a. No leaks. |  |
|  |  | b. Level alarms in order. |  |
|  |  | c. No leakages oil / water. |  |
|  |  | d. Starting batteries to be fully charged. |  |
|  |  | e. Mark required hydraulic pressure for starting on the gauge. |  |
|  |  | f. Fuel tank full. |  |
|  |  | g. Quick closing valve operational – Restore after testing Applicable to fuel tanks with 500 ltrs capacity or above. |  |
|  |  | h. Operating instructions posted in the Em. Gen. Room. |  |
| 20. | Fire Dampers | a. Not seized / corroded, clearly marked. |  |
|  |  | b. Control means easily operable (not stuck). |  |
| 21. | Emergency fire pump | a. Operate satisfactorily with adequate pressure (usually 5-7 bar). |  |
|  |  | b. Remote start in order. |  |
|  |  | c. Pressure gauges clear and in order. |  |
|  |  | d. Marked with IMO signals. |  |
|  |  | e. No leakages. |  |
|  |  | f. Operating instructions properly posted. |  |
|  |  | g. If vessel at sea in ballast condition, test the EFP prior arrival at port. |  |
| 22. | Water Mist System in the E/R | a. Ensure that supply valves are in “OPEN” position. |  |
|  |  | b. System is always on “AUTO” and not in “MANUAL” mode (if fitted). |  |
|  |  | c. Check the “AUTO” mode function for appropriate functioning (in accordance with maker’s Instructions). |  |

| F. Electrician |  |  |  |
| --- | --- | --- | --- |
| Item |  | Check | RATE: X= OK, N= Not Checked, N/A= Not applicable, R = REMARK – explain |
| 1. | Main/ Emergency Switchboards | a. Insulating mats available below all switchboards |  |
|  |  | b. No alarm of low insulation. |  |
|  |  | c. All light indication bulbs in order. |  |
|  |  | d. Emergency shutdowns (pumps, fans, etc.) tested/in order. |  |
|  |  | e. Preferential trips tested/in order |  |
|  |  | f. Instructions for restore normal operation after black out posted in engine control room. |  |
| 2. | Lighting/ Projectors | All lights/ covers and guards in good condition. |  |
| 3. | Emergency lights | Tested in good order. |  |
| 4. | Emergency Batteries | In good condition. Specific gravity of acid, liquid level and terminal voltage Maintenance records to be updated. Chargers in good condition operate satisfactorily (where fitted). |  |
| 5. | GMDSS Batteries |  |  |
| 6. | Emergency Generator Batteries |  |  |
| 7. | Lifeboat(s) batteries |  |  |
| 8. | Smoke/ Heat detectors | a. Remove any preventers of smoke detectors (if fitted by crew) |  |
|  |  | b. Check all smoke detectors for functioning. Have log all inspections and alarm tests. You must have onboard means to test the smoke detection. E.g., Cigarette lighters are NOT permitted to test Heat Detectors. |  |